# Work carried out at height (i.e. Work over two metres above ground/floor level)

## People at risk

| - Other workers/contractors in the vicinty - Client's personnel/employees/pupils | - Authorised and Unauthorised members of the public - Visitors |
| --- | --- |
| Hazard This column contains a comprehensive list of all the hazards that can be foreseen when this type of work is carried out. From knowledge of the task, omit any hazards that are not appropriate and take into account any other identified hazards that may be specific to the work situation/location. | ## Best Practice: All the precautions which are reasonably expected to be taken are listed here for each hazard. If the hazard is present, the law requires reasonable measures to be taken to reduce risk. Don’t consider anything that is irrelevant to the task in hand. Certain practices highlighted in bold print in this column are not merely guidance - they are specific legal requirements. |
| - Fire | Early warning of fire, eg automatic detection and alarm system (if they are present). Minimum of two alternative escape routes. Means of fighting fire to be available. Access for the Fire Brigade - have discussions with the Fire Brigade if access is to be restricted. Prepare a Fire Emergency Plan. Use flame-retardant sheeting on combustibles. Hot Works Permit to be used where applicable. |
| - Electrocution | Survey for overhead power lines and precautions in accordance with GS6 Avoidance of danger from overhead electrical lines. Consider proximity to live electrical cables in ceiling voids/roof spaces. |
| - Radiation, heat, exposure to fumes/microwave energy | Survey before working. Implement suitable controls in consultation with the owner of the source of the hazard. |
| - Fall of material | Establish and police exclusion zones. Wear hard hats. Rubbish chutes, fans and debris netting. Prohibit bombing or throwing of materials. |
| - Slips, trips and falls | Ensure safe access routes are provided. Good housekeeping and adequate lighting. Suitable footwear. Trained user of access equipment, no alcohol or substance abuse. Prohibit work in bad weather. Suitable edge protection or harnesses worn by trained wearers with inspection regime and tested anchorages. |

| ## Hazard (tick applicable) | ## Best Practice: |
| --- | --- |
| - Unauthorised access | Isolate/immobilise powered access equipment. Remove access ladders/provide physical barriers. |
| - Structural failure of access equipment. | Regular inspection, marking of safe working load and number of persons the equipment is capable of carrying safely. User inspection. Prohibit any adaptation or use for lifting materials. |
| - Falls from height or through fragile materials such as corrugated asbestos cement or rooflights, felt. | Physical barriers or covers to fragile areas. Permit to work. Identify unsafe areas by signs and barriers. |
| - Weather | Suspend work during strong winds, poor visibility or icy conditions. Provide chinstraps for safety helmets. |
| ## Relevant legislation and sources of information Management of Health and Safety at Work Regulations 1999 (SI 1999 No. 3242) (MHSWR) Provision and Use of Work Equipment Regulations 1998 (SI 1998 No. 2306) (PUWER) Lifting Operations and Lifting Equipment Regulations 1998 (SI 1998 No. 2307) (LOLER) Construction (Health, Safety and Welfare) Regulations 1996 (SI 1996 No. 1592) (C(HSW)R) Construction (Design and Management) Regulations 1994 (SI 1994 No. 3140) (CDM Regulations) Personal Protective Equipment at Work Regulations 1992 (SI 1992 No. 2966) HSG 33 Health and safety in roof work HSG 150 Health and safety in construction HSG 168 Fire safety in construction work GS31 Use of ladders, steps and trestles (out of print) BS 1129 Timber ladders, steps, trestles and lightweight stagings BS1129 Specification for portable timber ladders, steps, trestles and lightweight staging BS2037 Specification for portable aluminium ladders, steps, trestles and lightweight staging BS EN131 Ladders BS 5323 Code of Practice for Scissor Lifts BS 7171 Specification for MEWPs The Ladder User's Handbook DTI ref URN 95/577 Operators' Safety Guide for MEWPs, International Powered Access Federation |  |

| ## Training This section is to be used as a summary of competence standards that should be met by those carrying out the job. All workers to be trained on correct method of climbing, inspection, erection and lowering of any access equipment to be used. |  |
| --- | --- |
| ## Management This is a list of the monitoring activities and arrangements that management on site, in particular a principal contractor, will need to focus attention on to comply with CDM requirements. Daily monitoring by site manager. No ladder to exceed nine metres |  |
| ## Recommendations Consider other control measures that may be specified by the Responsible Premises Officer. |  |
| ## Other relevant risk assessments - Scaffolding - Mobil Elevated Work Platform Systems | - Ladders |